# Speech Anonymization for Dog Recordings — Progress Report (5–21 October 2026)

Oct 21, 2026 · @Nurmuhammed

## Summary

The anonymizer now runs without any expert labels and was always tested on participants its models never saw. Unprocessed, 19.8% of the 581 annotated human events contain words that Whisper can read. After the recommended detect-and-mute pipeline, 1.4% do, at a cost of 12.5% of the dog-only vocalization time.

- Detection was the bottleneck, and a modern audio model fixed most of it. EfficientAT (AudioSet, 2023) replaced VGGish (2017) as the frozen feature extractor. The detector's AUC rose from 0.80 to 0.91. At the strict setting it now finds 112 of the 133 events with readable speech, instead of 81.

- Muting beats subtracting. A fine-tuned speech remover (GTCRN) halved the leaks of the earlier remover. Even so, the hybrid (subtract only where a dog overlaps) never beat plain muting at equal dog loss.

- The numbers hold up. Choosing the thresholds without ever seeing the test person gives the same results. A counting bug in the Whisper check was found and fixed; all numbers here use the fixed rule.

- There is a ready-to-use command. It anonymizes any WAV file, mutes only inside the original file, and runs at about 19x real time on 2 CPU cores.

- Outside ELTE (synthetic tests): detection of foreign speech transfers (98–99.7% of the speech time is muted). Unfamiliar dogs are over-muted unless training includes other dogs. A privacy-first variant leaves 0 of 120 unseen utterances readable.

Current recommendation: EfficientAT (mn40) on the recording microphone, a logistic regression trained on the ELTE annotations, 5-frame smoothing, then mute. Code, tests and every result are in the repository (README Steps 12–21).

## Where we were on 5 October (Steps 1–11)

On 5 October the approach worked on synthetic mixtures but broke on the real ELTE BARKS recordings (30 play sessions, 17 participants, 581 annotated human events).

| Step | Finding |
| --- | --- |
| 1–4 | On synthetic mixtures, Silero VAD found the speech and a pretrained remover (DTLN) subtracted it by about 35 dB while keeping the dog. |
| 5 | On real recordings, Silero VAD found only 4.5% of the human time: the speech is quiet (53% below −60 dBFS), short and off-axis. |
| 6 | Even with the expert labels as a perfect detector, the removers took only 2–4 dB off the real human sounds (domain shift). 71% of the human time has no dog under it, so muting costs nothing there, which led to the hybrid (mute, and subtract only where a dog overlaps). |
| 7–8 | Privacy is checked independently with Whisper, against Whisper's own hallucination rate on background-only windows. |
| 9 | A detector trained on the lab's annotations (VGGish + logistic regression, leave-one-participant-out) reached AUC 0.80: 11x more human time found than Silero. |
| 10–11 | A labeling page for the speech tiers, and a dog detector (AUC 0.77) so the hybrid could run without labels. |

Agreed in the meeting: you would ask the head of department about using the dataset, Ivannova would focus on AI methods and TDoA, and the code would be shared on GitHub (done).

The privacy numbers in the 5 October report used Whisper word counts from before the fix described in Step 19. They are not directly comparable with the numbers below.

## Step 12: the full pipeline without expert labels

The trained detectors replaced the expert labels everywhere, so this is how a real anonymizer would run on a new recording. With the VGGish detector, detect-and-mute left 6.0% of human events readable at 9.0% dog loss (strict). It left 2.1% at 32.6% (balanced).

- The automatic hybrid with the pretrained remover (DTLN) was worse than muting at every setting. Subtraction on real far-field speech still leaks words.

- About 90% of the remaining leaks were events the detector had mostly missed (less than half of the event flagged). So detection, not removal, limited privacy. This set the direction for everything after.

- Each detector is run at the threshold where it flags a fixed share of the dog-only frames (3 / 6 / 10 / 15 / 25%). Two detectors are therefore always compared at the same cost to the dog.

## Step 14: a better speech remover (GTCRN, fine-tuned for the lab)

Replacing DTLN with GTCRN halved the speech left where a human overlaps a dog. Fine-tuning helped on simulated lab audio but not on the real events.

GTCRN (48k parameters, MIT licence) was fine-tuned on simulated far-field speech: LibriSpeech in a simulated 5.46 x 4.41 m room, mixed with real ELTE background at its real level. There were two models, one per participant group (odd / even), each tested only on the group it never saw.

| Remover | Synthetic: speech left | Synthetic: background kept | Real, oracle hybrid: readable events | ...of which overlap a dog |
| --- | --- | --- | --- | --- |
| DTLN (used before) | −2.7 dB | −3.2 dB | 7.1% | 16.7% |
| GTCRN, pretrained | −12.6 dB | −2.6 dB | 3.6% | 8.1% |
| GTCRN, fine-tuned | −15.8 dB | −1.7 dB | 3.6% | 8.1% |

- GTCRN vs DTLN on the same events: 31 leaks fixed and 11 new (McNemar p = 0.003). DTLN looked best on close-talk synthetic speech, but it is the worst on far-field speech.

- With the automatic detectors, the hybrid with this remover moved onto the same privacy / dog-loss curve as muting, but never above it. This still held with the better detectors of Step 16: 4 of 6 hybrid points were worse on both axes than some mute point, and the other 2 were trade-offs.

- (Readable shares in this table were counted with the rule before the Step 19 fix.)

## Steps 13 and 15: the three synchronized microphones

The dataset's raw recordings have three microphones per participant, all on one Zoom H4: M1 a Rode Wireless Go 2 clip-on, M2 the ME66 shotgun, M3 an ME64 cardioid. They helped the VGGish detector, but with the stronger model of Step 16 one microphone is enough.

- Alignment: every annotated session file was found inside its raw microphone file by cross-correlation. All 30 matched at correlation 1.000, at whole-second offsets, so the other microphones line up sample-exactly with the annotations.

- Spatial cues do not transfer. Band level differences and GCC-PHAT delays between the microphones gave AUC 0.66 alone and nothing on top of VGGish. People and dogs stand in different places in every session.

- Feeding all microphones into one input hurt (AUC 0.727): the model learned each training participant's setup.

- What worked was one detector trained on the frames of all microphones, run on each microphone, taking the maximum, then smoothing: AUC 0.797 → 0.837. In the pipeline, the same privacy cost about 40% less dog time (6% readable: 7.8% vs 12.7% dog loss).

- For the TDoA work: GCC-PHAT shows a sharp peak at lag 0 in every microphone pair, even with the wireless clip-on, which has its own transmission delay. This is electrical crosstalk inside the recorder, and it has to be masked before estimating delays.

## Steps 16–17: a modern audio model for detection (EfficientAT)

Replacing VGGish with EfficientAT was the largest single improvement. EfficientAT (Schmid et al., ICASSP 2023; MIT licence) is a small MobileNetV3 trained on AudioSet, which runs on a laptop CPU. The input is its 527 class scores, with the same logistic regression and leave-one-participant-out evaluation as before.

| Human detector | ROC AUC | Human time found at 6% dog-only frames flagged | ...at 25% |
| --- | --- | --- | --- |
| VGGish, session microphone (Step 9) | 0.797 | 50.0% | 70.0% |
| VGGish, three microphones (Step 15) | 0.837 | 56.7% | 78.2% |
| EfficientAT mn10, one microphone | 0.896 | 68.4% | 89.0% |
| EfficientAT mn40, one microphone | 0.912 | 72.1% | 90.8% |

outputs/detectors_pipeline/comparison.csv · corrected counting (Step 19) · one Whisper run, same 581 events

- Every EfficientAT point is significantly better than VGGish on the same events (exact McNemar, p ≤ 0.012).

- To reach 3% readable events, VGGish mutes 21.8% of the dog time and EfficientAT 5.3%. A participant bootstrap puts the saving at 14 points (95% CI 6–26).

- The dog detector improves too, from AUC 0.770 to 0.852.

- A technical trap: the model was trained on 10 s clips, and on 1 s windows its outputs saturate ("Chainsaw" at probability 1.0). The fix runs 10 s chunks through the convolutional part and pools the feature map per 0.96 s window. A unit test checks that this reproduces the model's own output.

## Step 18: a ready-to-use anonymization command

One command now anonymizes any WAV file with the recommended pipeline and needs no annotations:

python -m scripts.anonymize_recording input.wav anonymized.wav --strictness standard

- It mutes inside the original file, keeping its sample rate, bit depth and all its channels. The dog sound that is kept is bit-identical to the input. A .segments.csv lists what was muted.

- Speed: a 10.2-minute session takes 32 s on 2 CPU cores, including model loading. That is about 19x real time, or 3 minutes per hour of audio.

- Check: a model trained with P-02 left out, run through the command on P-02, reproduces the evaluation's predictions to within 0.008.

| Strictness | Threshold set at (dog-only frames flagged) | Readable events | Original words still readable | Dog time lost |
| --- | --- | --- | --- | --- |
| light | 6% | 2.2% | 9.6% | 7.9% |
| standard (default) | 10% | 1.4% | 5.6% | 12.5% |
| strict | 15% | 0.9% | 4.4% | 17.9% |

(Measured on participants the models never saw; unprocessed: 19.8% readable.)

The trained model file is 20 kB and is derived from the expert annotations. It stays out of the public repository until the dataset question is settled.

## Step 19: how reliable are the numbers?

Three checks: one found a bug in the privacy measure and fixed it, one confirmed that the thresholds were not tuned on the test data, and one ruled out a suspected cause of the remaining dog loss.

1. A counting bug, fixed. When Whisper's output stops in the middle of a sound caption, for example "(wind blowing" without the closing bracket, the caption was counted as spoken words. Reading every event still readable after the best pipeline exposed it. The text normalization now drops cut-off captions too (with a test), and every stored transcript was rescored.

|  | Before the fix | After the fix |
| --- | --- | --- |
| Unprocessed: human events with 2+ readable words | 22.9% | 19.8% |
| Whisper on background-only audio (the "floor") | 4.1% | 1.0% |
| VGGish + mute, 6% flagged | 7.9% | 6.0% |

The ranking of the methods does not change. Of the 8 events still readable at the standard setting, most are Whisper's typical hallucinations on sub-second clips ("Thank you.", "Mm-hmm.", Japanese or Korean fragments on Hungarian audio). Only about two contain plausible speech, and in one of them 2 of 17 words survive, none of them the original words.

2. Honest thresholds. Until now each threshold was chosen on the pooled out-of-fold predictions, which include the test participant's. In a nested version, an inner leave-one-out over the other 16 participants sets the threshold before it is applied to the 17th.

- The thresholds vary by only ±0.013 across folds, and 0.2–0.6% of frames are flagged differently.

- The pipeline gives the same results (e.g. standard: 1.4% readable, 12.6% dog lost).

- What does vary is the cost per recording: at a 3% target, the dog cost per participant ranges from 0.4% to 10.5%. The average is reliable; a single recording less so.

3. Finer cuts do not help. About a third of the lost dog time lies within 1 s of a human sound. Classifying the model's own 0.32 s feature-map columns instead of 0.96 s windows was slightly worse at every dog cost (human time muted at 10%: 76.9% vs 78.7%). The limit is detection itself, not time resolution.

## Steps 20–21: other speakers, other dogs, other rooms

The ELTE-trained detector finds foreign speech well but over-mutes unfamiliar dogs, and no single model is best everywhere. All earlier tests were ELTE: unseen people, but the same lab, microphones, language and kind of play. Here the models ran on synthetic 30 s scenes that differ in all of these:

- English read speech (LibriSpeech);

- ESC-50 dog clips (other dogs, other microphones, mostly outdoors);

- simulated rooms from 3 x 3 to 9 x 7 m, plus noise and varied recording levels.

Ground truth comes from the dry sources, and privacy is scored against the true text (an utterance leaks if Whisper gets 2+ of its words right). Speakers (20 / 20), dog clips (20 / 20) and random seeds are split between training and testing.

- Foreign speech: 99% of the speech time is muted at the standard level.

- Unfamiliar dogs: with no human present, the ELTE-only detector mutes 21–45% of their time (ELTE dogs: 8–18%). A smaller input (only the human and dog classes) does not fix this.

- Step 20 fix: adding scenes of 20 ESC-50 dogs as "not human" brings the 20 unseen clips down to 1.5–4.8%, with no change on ELTE. But it then stops muting speech that a bark covers.

- Step 21 fix: adding scenes with speech and dogs together, labelled by the true speech activity (speech under a bark is still speech), closes that gap.

| Model (standard level) | Held-out mixed scenes: utterances readable | Held-out mixed scenes: dog-only time muted | Unseen dogs alone: dog time muted | ELTE: readable events |
| --- | --- | --- | --- | --- |
| ELTE only (default) | 7.5% | 47% | 33% | 1.4% |
| + other dogs (Step 20) | 20.0% | 19% | 2.9% | 1.4% |
| + other dogs + speech under barks (Step 21) | 0% (0 of 120, at every level) | 72% | 15% | 2.1% |
| Silero VAD (generic) | 11.7% | 33% | 0% | finds little speech on ELTE |
| Mute the exact utterances (oracle) | 0% | 47% | 0% | — |

In the mixed scenes most dog-only time falls in pauses inside the utterances, so even muting exactly the utterances loses 47% of it. These are three points on one trade-off, kept as three model files:

- default: recordings like ELTE's;

- Step 21 model: unknown recordings where privacy comes first;

- Step 20 model: unknown recordings where keeping the dog matters more.

Caveat: the scenes are synthetic. The test speakers and dogs are unseen, but the scenes come from the same generator as the training scenes.

## What did not work

Each of these ideas was tested the same way (unseen participants, the same events) and left out:

| Idea | Result |
| --- | --- |
| Spatial cues between the three microphones (level differences, GCC-PHAT) | AUC 0.66 alone, no gain on top of the audio model |
| All three microphones concatenated into one input | Worse (AUC 0.727 vs 0.797) |
| Three microphones with the EfficientAT model | No gain over one microphone in the pipeline |
| A temporal CNN looking ±3.6 s around each frame | No better than logistic regression + 5-frame smoothing |
| Real 32 kHz audio cut from the 48 kHz raw files | Same as 16 kHz (AUC 0.918 vs 0.912) |
| Classifying 0.32 s feature-map columns for sharper cuts | Slightly worse at every dog cost |
| Only the human and dog AudioSet classes as input | Less overfitting to ELTE dogs, but did not fix it |
| Hybrid removal (subtract where a dog overlaps) | Never better than muting at equal dog loss, even with the fine-tuned GTCRN |

## Limitations, next steps and questions

Limitations

- The privacy check is automatic. It uses Whisper small, a strong but not the strongest recognizer, and no human listening test has been done yet.

- The training target is "every annotated human sound" (including laughs, claps, kissing sounds), not only intelligible speech.

- All real data come from one lab and 17 participants. The out-of-domain tests are synthetic.

- The dog cost differs a lot between recordings (0.4–10.5% at the same setting).

Next steps (proposed)

- A human listening audit of the remaining readable events (a labeling page is ready).

- A test on real recordings from another lab or another session type: the true out-of-domain check.

- More varied dog sounds in training (AudioSet / FSD50K clips) to make the general model keep more dog sound.

- A check with a stronger "attacker" (a larger Whisper model).

- A written report or paper draft of Steps 1–21.

Questions for you

- Is there news from the head of department about using the dataset? It decides whether the trained models can be published with the code.

- Which trade-off does the lab prefer: light (2.2% readable, 7.9% dog lost), standard (1.4%, 12.5%) or strict (0.9%, 17.9%)?

- Should the target be intelligible speech only, or every human sound?

- Do you know of other dog-human recordings (another lab, another setup) we could test on?

- Would you like this written up as a paper or a conference submission?

Code, tests (52 passing) and all results are in the GitHub repository; the README describes every step with the exact commands.